# Slides 5.10: Meet the Machine: Parts, Safety, Threading, and Paper Drills

Slide outline for Lesson 5.10, three days, twenty slides. Slides 1 to 8 are Day 1 (parts, rules, your machine). Slides 9 to 14 are Day 2 (threading and the license). Slides 15 to 20 are Day 3 (paper drills). No fabric appears on any slide on purpose; fabric starts in Lesson 5.11.

## Slide 1 (Day 1): Three seconds against twenty minutes

- Lesson 5.10, day one of three

- This machine sews in three seconds what your hands did in twenty minutes

- The trade: it does not know your finger is there

- These three days are how you earn the pedal Image: a machine-sewn seam and a hand-backstitched seam side by side, both four inches long.

## Slide 2: Do now, guess the seven parts

- Seven parts are numbered on the diagram

- Write what you think each one does. Guess.

- Star the one you think is the most dangerous Image: the numbered machine diagram from Handout 05.10, no labels.

## Slide 3: The seven parts

- 1 Needle: up and down, does not care what is under it

- 2 Presser foot: holds the fabric down

- 3 Feed dogs: pull the fabric through, so you guide, never push

- 4 Bobbin: the bottom thread, under the plate

- 5 Take-up lever: the arm the top thread rides

- 6 Hand wheel: one stitch at a time, always toward you

- 7 Foot pedal: the gas Image: the same diagram, labeled.

## Slide 4: The most dangerous part

- Most of you starred the needle. Fair.

- The better answer: the pedal

- The pedal moves the needle while a hand is in the wrong place Image: a foot resting beside (not on) a machine pedal.

## Slide 5: The four rules

- Hands to the sides of the needle, never in front. Guide, do not push.

- One foot on the pedal, the other flat on the floor

- Power off before changing a needle or reaching under the plate

- Hair back, sleeves clear, cords taped, nothing loose near the wheel

- The habit that is not a rule: hand wheel toward you, one stitch, look Image: two hands flat on fabric, one on each side of a presser foot, needle centered between them.

## Slide 6: Your machine, all unit

- The number on your machine is yours until the unit ends

- Same student, same machine, every day

- You will learn its habits. You will notice when something changed.

- Your threading buddy is the student at the next number Image: a numbered card taped to a sewing machine base, reading 4.

## Slide 7: Close-down, and the two hard rules

- Every machine day ends the same way: presser foot down, power off, chair in

- A broken needle is reported and handed over, both pieces, flat on your palm

- Never the trash. Never the floor.

- A jammed machine gets the red doctor tag: tag it, tell me, move to a spare or the hand station, keep working Image: a red MACHINE DOCTOR tag hanging from a machine’s arm.

## Slide 8 (Day 1 close): Prove it

- Run the close-down routine on your machine, out loud

- Exit line at the door: one part and what it does Image: a row of machines, feet down, a pushed-in chair at each.

## Slide 9 (Day 2): Do now, the path from memory

- Write the top threading path in order, as far as you remember

- Do not look at the handout yet Image: a spool of thread alone on a table.

## Slide 10: Threading the top, six steps

- Spool on the pin

- Through the first guide

- Down and around the tension dial

- Up through the take-up lever, back to front

- Down through the lower guides

- Through the needle, front to back Image: the threading path drawn as a single line over the machine diagram, numbered.

## Slide 11: The bobbin

- Drop the wound bobbin in the case the way the arrow shows

- Thread through the slot, close the plate

- Hold the top thread, hand wheel one full turn toward you

- Pull the loop up, both threads back under the foot Image: a bobbin case with an arrow showing thread direction, and a loop of bobbin thread coming up through the needle plate.

## Slide 12: The manual outranks me

- Every machine is a little different

- The manual at your machine is the authority

- When the diagram and your machine disagree, the manual wins Image: a machine manual open in its binder beside a machine.

## Slide 13: The threading license

- Thread the top and load the bobbin at your own machine

- Your buddy checks every step against the guide and initials

- I check the tension and the bobbin, and initial

- Two initials make a license: from then on you sit at a machine you threaded Image: the license line from Handout 05.10 with two sets of initials.

## Slide 14 (Day 2 close): Unthread it

- Finished early? Unthread the whole machine and thread it again from nothing

- That is the real test

- Close-down routine, then the exit line: the step you will forget tomorrow Image: a machine with thread removed, spool sitting beside it.

## Slide 15 (Day 3): Do now, rules from memory

- Write the four rules without looking

- Then write the close-down routine Image: blank lines numbered 1 to 4, then 1 to 3.

## Slide 16: Paper, no thread

- Today the machine has no thread at all

- The needle punches holes, and the holes are your record

- Hold the paper up to the light: every wobble shows

- This is how factories train, and it costs nothing Image: a sheet of paper with a drawn line and a row of needle holes running almost exactly along it.

## Slide 17: The three sheets

- Sheet 1: the straight line, three passes

- Sheet 2: the curve, slow and steady, small steering

- Sheet 3: the square, with a pivot at each corner Image: the three drill sheets fanned out.

## Slide 18: The pivot

- Stop with the needle DOWN in the paper

- Foot UP

- PIVOT the paper around the needle

- Foot DOWN, keep going

- Say it as a chant: needle down, foot up, pivot, foot down Image: a square drill sheet mid-turn, needle down at the corner, presser foot lifted.

## Slide 19: What I am checking

- Hands to the sides

- Steady speed

- Holes on the line

- Cannot keep hands to the sides? You drive with the hand wheel until you can. That is a technique, not a penalty. Image: the three-box clipboard checklist.

## Slide 20 (Day 3 close): Trade and mark

- Trade your square with your buddy

- Mark their straightest two inches and their wobbliest two inches

- Square sheets come to me: they are the record that the license is real

- Close-down routine. Tomorrow, fabric. Image: two drill squares side by side, two sections circled on each.