# Handout 4.10: Floor Plan Symbols, Graph Paper Guide, and Furniture Cut-Outs

Name: ______________________________ Date: ______________ Team: ______

Directions: Fill in Part 1 while I draw the symbols on the board. Part 2 is the scale rule. It stays in your project folder all unit. Part 3 is the record sheet your team fills in with the tape measure. Part 4 is the cut-out sheet. Cut only the pieces your room needs and keep the rest. Part 5 has the rules every path has to pass. It also has the list of the eleven sketch prompts, so you always know what is coming. Pencil only. This whole handout lives in the project folder.

## Word bank in Spanish

From the unit vocabulary list.

| English | Spanish |
| --- | --- |
| floor plan | plano de planta |
| scale | escala |
| traffic flow | flujo de circulación |
| zone | zona |
| tape measure | cinta métrica |
| universal design | diseño universal |

## Part 1: The symbol legend

A floor plan is a room drawn from straight above. Everything in it is a symbol. The symbols are the same on every plan in the country. That is why a stranger can read your drawing. Draw each one in the box.

| Symbol | How to draw it | Draw it here |
| --- | --- | --- |
| Wall | Two parallel lines close together, filled in dark. Thin lines for an inside wall, a little thicker for an outside wall. |  |
| Door | Leave a gap in the wall the width of the door. Draw a straight line for the door panel standing open, and a quarter-circle arc from the end of that line back to the wall. The arc is the swing. |  |
| Window | Leave a gap in the wall, then draw two or three thin lines across the gap, parallel to the wall. |  |
| Closet | A small rectangle drawn off the room, open to the room. A straight line across the opening for a sliding or bifold door, a dashed line for a curtain, or a door with a swing arc if it has one. |  |
| Stairs | A run of short parallel lines like a ladder, inside a long rectangle, with an arrow through the middle and the word UP (or DN for down). |  |
| Sink | A rectangle with a small circle in the middle for the drain. A kitchen sink is a rectangle with two circles. |  |
| Toilet | A small rectangle against the wall for the tank, with an oval sticking out from it into the room. |  |
| Tub | A long rectangle against the wall, one end rounded, with a small circle at the square end for the drain. |  |
| Stove (range) | A square with four small circles in it for the burners. |  |
| Refrigerator | A square or a rectangle with the letters REF written inside it. |  |
| Outlet | A short line drawn on the wall with two small dots beside it. Draw it on the inside of the wall line. |  |

Reading a real plan (Day 1). Find each of these on the plan I put up and circle it. Then answer.

[ ] the front door, and which way it swings [ ] a window [ ] the bathroom [ ] the kitchen [ ] a closet

How do you know which room is the bathroom without reading a label? _______________________________

How do you know which room is the kitchen? ___________________________________________________

Draw the path a person walks from the front door to the kitchen sink. What does the path pass through? ____________________________________________________________________

The four zones. Every room gets split into jobs. Label them on the plan.

| Zone | What happens there | Where it usually goes |
| --- | --- | --- |
| Sleep |  |  |
| Work |  |  |
| Store |  |  |
| Move |  |  |

## Part 2: How to use the graph paper

The scale for this lesson is 1/4 inch = 1 foot.

Every square on your graph paper is 1/4 inch across. So at this scale:

One square = one foot. That is the whole rule.

Worked conversion 1. A bedroom wall is 12 feet long. How long is the line on the paper?

12 feet x 1 square per foot = 12 squares. 12 squares x 1/4 inch = 12/4 = 3 inches of line on the paper.

Worked conversion 2. A path has to be 36 inches wide. How many squares is that?

36 inches ÷ 12 inches per foot = 3 feet. 3 feet x 1 square per foot = 3 squares.

Two more you should be able to do in your head by tomorrow:

A 10-foot wall is __________ squares, which is __________ inches on the paper.

30 inches is __________ feet, which is __________ squares (one of them is a half square).

Four rules for every plan you draw this unit:

- Write the scale in the bottom corner of the sheet. Every plan. No exceptions.

- Write whose room it is at the top.

- Use a ruler for every wall. A wall drawn freehand is a wall nobody can measure.

- Pencil only, because you will move something.

Rounding. Measure to the nearest half foot. Then draw to the nearest whole square. Rounding to the nearest square is close enough at this scale. It also keeps you on the grid lines. In Lesson 4.12 the scale doubles to 1/2 inch = 1 foot. That is two squares per foot. There a half foot is one whole square, so you can be exact.

If graph paper runs out: rule a grid of quarter-inch squares on plain paper with a ruler and keep the same scale.

## Part 3: Measuring record, our classroom

Each team measures the walls it is assigned. Hook the end of the tape in the corner. Read the tape to the nearest half inch, then round to the nearest half foot and write both numbers.

| What we measured | Tape reading (feet and inches) | Rounded (feet, to the nearest half foot) | Squares at 1/4 inch = 1 foot |
| --- | --- | --- | --- |
| Wall A (the wall with the door) |  |  |  |
| Wall B |  |  |  |
| Wall C |  |  |  |
| Wall D (the wall with the window) |  |  |  |
| Width of the doorway |  |  |  |
| Corner to the near edge of the door (along wall A) |  |  |  |
| Width of window 1 |  |  |  |
| Corner to the near edge of window 1 |  |  |  |
| Width of window 2 (if there is one) |  |  |  |
| Height of the ceiling (optional, and you need help for this one) |  |  |  |

Class numbers we agreed on: A ______ ft, B ______ ft, C ______ ft, D ______ ft

Check before you draw: do walls A and C match? Do B and D match? If they do not, this room is not a rectangle, which is normal, and your plan should show the shape it really is.

One thing that was harder to measure than I expected: ____________________________________

## Part 4: Furniture cut-out sheet (1/4 inch = 1 foot)

Every piece below is drawn at the same scale as your plan. Cut only what your room needs. Label each piece before you cut it. A blank rectangle is easy to mix up.

How to read this table. The real size is the size of the real furniture, in inches, the way a store lists it. The feet column is that size divided by 12. The squares column is what you draw and cut. It is rounded to the nearest whole square. Where the rounding moved the size by half a foot or more, the note says which way. On a tight plan, six inches matters.

| # | Piece | Real size (inches) | In feet | Squares to cut (wide by long) | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | Bed, twin | 39 by 75 | 3.25 by 6.25 | 3 by 6 | The real bed is a few inches bigger than the cut-out. Leave a little extra |
| 2 | Bed, twin XL | 39 by 80 | 3.25 by 6.67 | 3 by 7 |  |
| 3 | Bed, full (double) | 54 by 75 | 4.5 by 6.25 | 5 by 6 | Rounded up on the width |
| 4 | Bed, queen | 60 by 80 | 5 by 6.67 | 5 by 7 |  |
| 5 | Bunk bed, twin over twin (floor space) | 41 by 78 | 3.4 by 6.5 | 3 by 7 | Takes one floor space, sleeps two. Needs headroom, not floor |
| 6 | Crib | 28 by 52 | 2.3 by 4.3 | 2 by 4 |  |
| 7 | Nightstand | 18 by 18 | 1.5 by 1.5 | 2 by 2 | Rounded up both ways |
| 8 | Dresser, 6 drawer (long) | 60 by 18 | 5 by 1.5 | 5 by 2 | Rounded up on the depth |
| 9 | Dresser, 3 drawer | 32 by 18 | 2.7 by 1.5 | 3 by 2 |  |
| 10 | Desk, student | 48 by 24 | 4 by 2 | 4 by 2 | Exact |
| 11 | Desk, small or wall fold-down | 36 by 20 | 3 by 1.7 | 3 by 2 |  |
| 12 | Desk chair | 18 by 18 | 1.5 by 1.5 | 2 by 2 | Needs 30 inches behind it to pull out, which is 2.5 squares of path |
| 13 | Armchair with arms | 30 by 30 | 2.5 by 2.5 | 3 by 3 |  |
| 14 | Sofa, 3 seat | 84 by 36 | 7 by 3 | 7 by 3 | Exact |
| 15 | Loveseat, 2 seat | 60 by 36 | 5 by 3 | 5 by 3 | Exact |
| 16 | Bookcase, 3 shelf | 30 by 12 | 2.5 by 1 | 3 by 1 |  |
| 17 | Cube storage, 9 cube | 36 by 15 | 3 by 1.25 | 3 by 1 |  |
| 18 | Wardrobe or armoire | 48 by 24 | 4 by 2 | 4 by 2 | Use this if the room has no closet |
| 19 | Storage bench or toy bin | 36 by 18 | 3 by 1.5 | 3 by 2 |  |
| 20 | TV on a stand | 60 by 16 | 5 by 1.3 | 5 by 1 |  |
| 21 | Floor lamp | 12 by 12 | 1 by 1 | 1 by 1 | Exact |
| 22 | Dining table, round, seats 4 | 42 across | 3.5 across | 4 by 4, drawn as a circle | Rounded up. Each chair needs 2 by 2 and 30 inches to slide back |
| 23 | Dining table, rectangle, seats 6 | 72 by 36 | 6 by 3 | 6 by 3 | Exact |
| 24 | Dining or side chair | 18 by 18 | 1.5 by 1.5 | 2 by 2 |  |
| 25 | Area rug, 5 by 7 | 60 by 84 | 5 by 7 | 5 by 7, drawn with a dashed outline | A rug goes under furniture, so draw it first and dashed |

Grade 8 stretch: pick any three pieces and write their real footprint in square feet (width in feet times length in feet). Then add the three and say what percent of your room’s floor they cover.

## Part 5: Traffic flow rules, the described bedroom, and the sketch prompts

### The traffic flow rules

Traffic flow is the path people really walk. Furniture never blocks it. Shade every path on your plan with a pencil. Then measure the narrowest point of that path, not the entrance. A path is only as wide as its tightest spot.

| The rule | In inches | In feet | Squares at 1/4 inch = 1 foot |
| --- | --- | --- | --- |
| Any path a person using a walker or a wheelchair uses, and the front entry | 36 inches | 3 feet | 3 squares |
| Every other path in a room | 30 inches | 2.5 feet | 2.5 squares |
| A circle a wheelchair needs to turn all the way around | 60 inches across | 5 feet | 5 squares across |
| Behind a chair so a person can push it back and stand up | 30 inches | 2.5 feet | 2.5 squares |

Three more rules that are not numbers:

- The door swing stays clear. Nothing parks in the arc.

- Nothing goes in front of a window that has to open, and nothing blocks a window that is an escape route from a bedroom.

- A bed needs at least one open side, so a person can get in and out of it and somebody can change the sheets.

Where the numbers come from: 36 inches and the 60-inch turning circle are from the 2010 ADA Standards for Accessible Design, sections 403.5.1 and 304.3. The 30-inch general path is a common planning guideline, not a code line.

### The described bedroom (use this on Day 2 if you would rather not draw your own)

A bedroom, 11 feet by 12 feet.

- One door, 3 feet wide, on a short wall, 2 feet from the corner, swinging into the room.

- One window, 4 feet wide, centered on the far long wall.

- One closet, 2 feet deep by 5 feet wide, on the other long wall, with sliding doors.

- The person who sleeps here is a middle schooler who does homework in the room.

Draw it to scale, then place a bed, one storage piece, and one more piece the room needs. Pencil the four zones. Shade every path, measure the narrowest point, and fix the one that fails.

What I moved, and why: ____________________________________________________________

### The eleven sketch prompts (the daily floor plan do now)

Starting on day 104 and running to the last build day, every period opens with a five-minute floor plan sketch. You get two minutes to draw and one minute to check with a neighbor. Then the sheet goes in your project folder. That makes eleven sheets. Staple them in order with your name on the first sheet. Turn the stack in with the project. Each sheet is scored complete, partial, or missing. If you are absent, the prompt is on this list, so you can draw it and add it to the stack.

| # | Lesson | The prompt |
| --- | --- | --- |
| 1 | 4.10 Day 1, do now | This classroom from straight above, with the door, the windows, and the teacher’s desk. No scale yet |
| 2 | 4.10 Day 1, closure | This room again with the desks arranged a different way, and one path from the door to the window shaded |
| 3 | 4.10 Day 2, do now | A 10 by 11 bedroom at 1/4 inch = 1 foot, a 3-foot door with its swing on a short wall, a 4-foot window on a long wall, scale labeled |
| 4 | 4.10 Day 2, closure | Any room in this building from above, to scale, with one path shaded, plus one line: the thing a floor plan can prove that a photograph cannot |
| 5 | 4.11 | A 10 by 12 room at 1/4 inch = 1 foot with a door and a window, using the symbols |
| 6 | 4.12 Day 1 | A 12 by 14 room redrawn with the bed against a wall and a straight path from the door to the window |
| 7 | 4.12 Day 2 | A 6 by 9 entry with the front door on a short wall, a bench and hooks, the path shaded and measured. Does it pass 36 inches? |
| 8 | 4.13 | Your client’s room from memory, no looking at your plan |
| 9 | 4.14 Day 1 | Your client’s room as a folded-down net (the floor in the middle, the four walls folded out flat) |
| 10 | 4.14 Day 2 | One wall of your client’s room as an elevation (straight on, not from above) at 1/2 inch = 1 foot |
| 11 | 4.14 Day 3 | Any room with a 5-foot turning circle drawn in it, and the circle has to fit |